| DENIM UNIVERSE STANDARD OPERATING PROCEDURE & TECHNICAL MANUAL Industrial Denim Engineering Â· Wet Processing Â· Quality Governance | DOCUMENT CONTROL REF DOC ID: DU-SOP-2026-001 DEPARTMENT: Wet Processing / Dyeing STATUS: APPROVED & ACTIVE (REV 2.4) EFFECTIVE: 2026-09-22 Â· REVIEW: ANNUAL |
| --- | --- |

OFFICIAL TECHNICAL PROCEDURE & PRODUCTION SPECIFICATION [INSERT YOUR SOP TITLE HERE â€” E.G. CONTINUOUS ROPE DYEING & INDIGO REDUCTION PROTOCOL] Scope: Standardized operational sequence, chemical dosing, redox stabilization, laboratory tolerance verification, and defect mitigation for industrial denim production.

| PLANT / PRODUCTION UNIT | Denim Mill Unit 01 (Wet Mill) | PROCESS OWNER | Senior Dyeing Technologist |
| --- | --- | --- | --- |
| APPLICABLE MACHINERY | Morrison / Master Rope Range | QUALITY BENCHMARK | 4-Point System / ASTM D1424 |
| CHEMICAL COMPLIANCE | ZDHC MRSL v3.1 Level 3 | DISTRIBUTION LEVEL | Floor Supervised / Confidential |

# 1. PURPOSE, SCOPE & FACTORY SAFETY MANDATE

This Standard Operating Procedure (SOP) defines the mandatory operational parameters, machine setup conditions, chemical replenishment formulations, and in-line quality controls for [insert process name, e.g. Rope Dyeing of 100% Cotton Indigo Warp Yarn]. Strict adherence to this document is compulsory for all shift engineers, section officers, machine operators, and quality controllers to ensure batch-to-batch shade reproducibility, optimal fabric tear strength, and zero non-conformance.

CRITICAL SAFETY, CHEMICAL HYGIENE & ZDHC COMPLIANCE â€¢ Mandatory Personal Protective Equipment (PPE): Full chemical face shield (ANSI Z87.1), elbow-length nitrile/neoprene gloves, alkali-resistant rubber safety boots with steel toe, and vapor cartridge respirator (EN 14387 A1) when handling caustic soda (NaOH) or sodium hydrosulfite. â€¢ Chemical Hygiene & Dissolving: Never add water to concentrated caustic flakes; always add caustic slowly to water under continuous mechanical stirring. Sodium hydrosulfite must be stored in airtight containers away from moisture and heat sources. â€¢ Emergency Eye-Wash & Shower Protocol: In the event of accidental skin or eye contact with dye liquor or caustic liquor, immediately flush under the nearest deluge shower/eyewash unit for at least 15 continuous minutes and trigger the medical emergency code.

# 2. MACHINE SETUP & TECHNICAL PARAMETERS MATRIX

The following baseline parameters must be configured on the central control console prior to launching continuous lot processing. Any deviation beyond the Upper or Lower Specification Limits triggers an immediate automated halt or mandatory corrective dosing.

| Process Parameter | Standard Target | Min Limit | Max Limit | Check Interval | Measuring Tool |
| --- | --- | --- | --- | --- | --- |
| Pre-Scouring Temp (Â°C) | 65 Â°C | 60 Â°C | 70 Â°C | Continuous / Hourly | PT100 Temp Sensor |
| Caustic Soda (NaOH) | 38 Â°BÃ© | 36 Â°BÃ© | 40 Â°BÃ© | Every 2 Hours | BaumÃ© Hydrometer |
| Indigo Dye Stock (g/L) | 4.50 g/L | 4.20 g/L | 4.80 g/L | Shift Start / Refill | Spectrophotometer |
| Sodium Hydrosulfite (Hydro) | 3.20 g/L | 2.80 g/L | 3.60 g/L | Every 2 Hours | Redox Titration |
| Dye Bath Redox Potential | -780 mV | -750 mV | -820 mV | Continuous Online | ORP Electrode |
| Dye Bath pH Level | pH 12.2 | pH 11.8 | pH 12.6 | Hourly | Digital pH Meter |
| Oxidation Skying Time (s) | 120 sec | 110 sec | 130 sec | Per Speed Shift | Tachometer / Speed |
| Squeezer Nip Pressure | 3.5 bar | 3.2 bar | 3.8 bar | Every 2 Hours | Pneumatic Gauge |
| Moisture Regain at Doff | 6.5 % | 5.5 % | 7.5 % | Head & Tail of Beam | Aqua-Boy Meter |

# 3. STANDARD OPERATING WORKFLOW (STEP-BY-STEP)

Phase 3.1: Pre-Operational Machine Inspection & Line Clearance

1. Inspect all squeeze roller surfaces for grooves, scratches, or foreign matter accumulation. Verify uniform nip contact using carbon contact test paper across the 1800mm working face.

2. Ensure all overflow weir gates, filter screens, and chemical circulation pumps are clear of sediment and dried indigo crust. Verify calibration date on all inline pH and ORP probes.

Phase 3.2: Chemical Stock Preparation & Auto-Dosing Calibration

1. Prepare concentrated stock leuco-indigo vat in designated dissolution tank with mechanical agitator running at 350 RPM. Confirm complete reduction (bright yellow-green liquor with bronze-purple surface sheen).

2. Transfer stock liquor to machine replenishment tanks. Prime metering dosing pumps and calibrate strokes per minute to match yarn throughput speed and recipe consumption curve.

Phase 3.3: Continuous Running, Immersion & Skying Oxidation

1. Thread leader yarn through pre-treatment, dye vats (typically 8 to 12 dips), skying frames, and wash boxes. Gradually ramp speed up to specified target rate (e.g. 28 - 32 m/min).

2. Observe skying oxidation zone: ensure complete transition from leuco-yellow to vibrant blue occurs within the first 60% of vertical travel prior to entry into the subsequent dye dip.

Phase 3.4: Post-Washing, Neutralization & Cylinder Drying

1. Run counter-current washing with freshwater cascade to strip unfixed surface dye molecules. Maintain Box 1 & 2 at 60Â°C, Box 3 (acid neutralization with acetic acid) at pH 6.0 - 6.5, Box 4 at ambient temperature.

2. Dry yarn uniformly across steam-heated Teflon-coated cylinder stacks. Regulate cylinder steam pressure (2.0 to 2.8 bar) to hit delivery moisture regain between 5.5% and 7.5%.

Phase 3.5: Doffing, Batch Identification & Lab Sampling

1. Cut and secure yarn ropes or fabric rolls onto take-up beams. Affix moisture-resistant barcode lot identification label indicating Machine No, Lot ID, Yarn Count, Shade Code, Gross Weight, and Operator ID.

2. Extract 2.0-meter head swatch and tail swatch from each beam/roll. Send immediately to Quality Control Physical Testing Laboratory for spectrophotometric and dimensional testing.

# 4. QUALITY CHECKPOINTS & LABORATORY ACCEPTANCE MATRIX

Every completed production roll or beam must comply with the acceptance criteria outlined below. Lots exceeding critical tolerance thresholds are instantly routed to Material Quarantine for disposition by the Chief Metallurgist / Technical Director.

| Inspection Checkpoint | Test Standard | Acceptance Tolerance | Frequency | Action on Fail |
| --- | --- | --- | --- | --- |
| Shade Consistency (CIE DEcmc) | ASTM D2244 / ISO 105-J03 | DEcmc â‰¤ 0.80 vs Standard | Every 1,000m / Doff | Quarantine & Shade Group |
| Fabric Weight (GSM / oz) | ASTM D3776 / ISO 3801 | Target Â± 3.0% (e.g. Â±0.4 oz) | Head & Tail Each Roll | Adjust Loom Weft Tension |
| Residual Warp Shrinkage | AATCC 135 (3 washes) | -1.5% to -2.5% Maximum | 1 test per 3,000m | Re-Sanforize Roll |
| Weft Skewness / Bowing | ASTM D3882 | Skew â‰¤ 2.0% ¦ Bow â‰¤ 1.5% | Continuous Online | Adjust Weft Straightener |
| Color Fastness to Rubbing | AATCC 8 / ISO 105-X12 | Dry: Grade 4.0 ¦ Wet: Grade 2.5 | 1 test per dye lot | Re-Scour Wash Line |
| Tensile & Tear Strength | ASTM D5034 / D1424 | Warp > 700 N ¦ Weft > 450 N | 1 test per master lot | Audit Yarn Ring Spinning |

# 5. TROUBLESHOOTING & CORRECTIVE ACTION MATRIX

When operational defects emerge, the floor technician must reference this cause-and-effect matrix to execute immediate containment followed by root-cause Corrective and Preventive Action (CAPA).

| Observed Defect / Symptom | Probable Root Causes | Immediate Containment | Permanent CAPA |
| --- | --- | --- | --- |
| Side-Center Shade Variation (Listing) | Uneven squeezer roll hardness; differential pneumatic pressure left vs right; clogged bath recirculation spray pipes. | Inspect cylinder pressure gauges; re-balance left/right air regulators; flush spray headers. | Schedule precision roll grinding with crown profile; calibrate pressure transducers monthly. |
| Bronzing / Unfixed Surface Indigo | Insufficient sodium hydrosulfite (low redox potential); premature air oxidation in dye vat. | Immediately add 0.5 g/L hydro shock dose to restore ORP < -780 mV. | Inspect nitrogen blanket injection or check pump mechanical seal integrity to prevent air suction. |
| Excessive Warp Shrinkage After Wash | Rubber sanforizing belt wear; inadequate steaming / moisture conditioning on entry. | Increase rubber belt compressive shrinkage overfeed by 1.5%; raise damping steam pressure. | Schedule rubber belt re-grinding; verify steam nozzle cleanout every weekly maintenance window. |
| Skewness / Leg Twist in Garment | Incorrect optical weft straightener angle; differential web tension in finishing cylinders. | Manually override weft straightener angle to compensate twill directional torque. | Recalibrate optical Mahlo sensors and servo positioning motors across all fabric styles. |

# 6. DOCUMENT REVISION HISTORY & APPROVAL LOG

| Rev # | Release Date | Description of Technical Change / Amendment | Approved By |
| --- | --- | --- | --- |
| v1.0 | 2024-03-15 | Initial formal factory baseline release for industrial denim wet processing. | Technical Operations |
| v2.0 | 2025-08-10 | Updated ZDHC MRSL v3.1 chemical screening criteria & ORP auto-dosing loop. | Quality Committee |
| v2.4 | 2026-09-22 | Integrated editable digital template parameters and ASTM D3882 skewness limits. | Engr. Asif Jahan |

# 7. OFFICIAL AUTHORIZATION & SIGN-OFF SIGNATURES

By signing below, the designated mill leadership affirms that this procedure has been validated against production trials, safety standards, and customer chemical restrictions. Controlled document under Denim Universe Quality Management System.

| PREPARED & COMPILED BY Name: Engr. Asif Jahan Role: Senior Textile Process Engineer Date: 2026-09-22 Signature: ______________________ | REVIEWED & VERIFIED BY Name: [QA Manager Name] Role: Head of Quality Assurance Date: [Insert Date] Signature: ______________________ | FINAL APPROVAL & RELEASE Name: [General Manager Name] Role: General Manager (Operations) Date: [Insert Date] Signature: ______________________ |
| --- | --- | --- |

DENIM UNIVERSE QUALITY ASSURANCE SYSTEM Â· ACCREDITED STANDARD ¦ ISO 9001:2015 Â· ZDHC MRSL v3.1 Â· OEKO-TEX STANDARD 100 Â· HIGG FEM